i36
BE5010 Dual-Band WiFi7 Ceiling Access Point

| Product Information |  |
| --- | --- |
| Model | i36 |
| Category | Ceiling |
| Dimension | Φ250.6*55 mm |
| Hardware |  |
| Interface | 1*100/1000/2500Mbps Ethernet Port (PoE) 1*10/100/1000Mbps Ethernet Port |
| Button | 1*Reset |
| LED Indicator | 1*SYS |
| Maximum Power Consumption | 17.4W |
| Power Supply | PoE 802.3at |
| Maximum number of associated wireless clients | 256 |
| Recommended Clients | 100 |
| Antenna Gain | 4dBi |
| 2.4GHz maximum power | • CE: <20 dBm (2.4 GHz, EIRP) • FCC: <29 dBm (2.4 GHz) |
| 5GHz maximum power | • CE: <23 dBm(5 GHz, EIRP) • FCC: <31 dBm(5 GHz) |
| Reception Sensitivity | 2.4GHz 802.11b -98dBm 2.4GHz 802.11n (MCS7) -77dBm 5GHz 802.11a -95dBm 5GHz 802.11n (40M-MCS7) -76dBm 5GHz 802.11ax (80M-MCS11) -64dBm |
| Frequency Range | 2.4GHz&5GHz |
| Standard&Protocol | IEEE 802.11a/b/g/n/ac/ax/be |
| Maximum wireless speed | 2.4GHz: 688Mbps 5GHz: 4323Mbps |
| Bandwidth | 2.4GHz: Auto/20/40 MHz 5 GHz: Auto/20/40/80/160 MHz |
| Software |  |
| Operating Mode | AP/Client+AP |
| Max. No. of SSID | 2.4GHz: 8 5GHz: 4 |
| Hide SSID | Supported |
| OFDMA | Supported |
| Beamforming | Supported |
| Fast roaming（802.11k/v/r） | Supported |
| WEP | Supported |
| WPA-PSK | Supported |
| WPA2-PSK | Supported |
| WPA | Supported |
| WPA2 | Supported |
| WPA3-ASE | Supported |
| Access control | Supported |
| Adjustable power transmit | Supported |
| Automatic channel | Supported |
| AP isolation | Supported |
| Guest network | Supported |
| Connected clients control | Supported |
| RSSI Threshold | Supported |
| Weak signal terminal elimination | Supported |
| Prioritize 5 GHz | Supported |
| VLAN tagging for SSID | Supported |
| LED control | Supported |
| MU-MIMO | Supported |
| Diagnostics tool | Supported |
| Reboot at specified interval | Supported |
| Schedule Reboot | Supported |
| Device management | Local Web Management Tenda CloudFi Cloud Management Tenda CloudFi APP Management Tenda Gateway Management（G300-F\G0-8G-PoE\G0-5G-PoE\G1） |
| System Logs | Supported |
| Firmware upgrade | Supported |
| Reboot | Supported |
| Reset | Supported |
| Backup configuration | Supported |
| Restore configuration | Supported |
| Others |  |
| Default address | 10.16.16.169 192.168.0.254 |
| Operation Temperature | -10℃ ~ 40℃ |
| Operating humidity | 10%~90% non-condensing |
| Storage temperature | -30℃ ~ 70℃ |
| Storage humidity | 5%~90% non-condensing |