ME3 Pro 3-Pack
BE3600 Whole Home Mesh Wi-Fi7 System

| Hardware Features |  |
| --- | --- |
| Standard&Protocol | IEEE 802.3, IEEE 802.3u, IEEE 802.3ab |
| CPU | Dual-Core 1GHz CPU |
| RAM | 512MB |
| Interface | 3 x GE ports per node |
| Antenna | 5 x 3dBi Internal antennas |
| Button | 1 x RESET Button 1 x MESH Button |
| Power | Input: AC 100-240V, 50/60Hz Output: DC 12 V ⎓ 2 A |
| LED indicator | 1 x Tri-color LED Indicator(Green,Yellow,Red) |
| Dimensions | 86*86*190mm (3.39 *3.39 * 7.48 in) |
| Wireless Features |  |
| Wireless Standards | 5GHz: IEEE 802.11be/ax/ac/n/a 2.4GHz: IEEE 802.11be/ax/n/b/g |
| Data Rate | 2.4GHz: 688Mbps (802.11be) 5GHz: 2882Mbps (802.11be) |
| Frequency | 2.4GHz & 5GHz |
| Transmission Power | CE: <20 dBm (2.4 GHz) <23 dBm (5 GHz) FCC: <30 dBm (2.4 GHz) <30 dBm (5 GHz) |
| Basic Functions | Beam Forming MU-MIMO AP Steering Band Steering OFDMA 4K-QAM |
| Wireless Security | WPA2-PSK WPA3-SAE/WPA2-PSK |
| Software Function |  |
| Internet Connection Type | PPPoE, Dynamic IP, Static IP PPPoE(Russia), PPTP(Russia), L2TP(Russia) Unify,Maxis,Maxis-special,Celcom,Digi |
| Network Protocol | IPv4&IPv6 |
| Operating Mode | Router Mode,AP Mode,WISP Mode,Client+AP Mode |
| Parental Control | Client Filter URL Filter Time Limit |
| DHCP | DHCP Server DHCP Client List |
| Virtual Server | Port Forwarding DMZ Host UPnP |
| Firewall | Forbid UDP Flood Attack Forbid TCP Flood Attack Forbid ICMP Flood Attack Forbid WAN Ping from Internet |
| DNS | DynDNS, no-ip, oray, 88ip |
| VPN | PPTP Server, PPTP/L2TP Client |
| Special Features | WPA3 TR069 Unify 2.4 GHz & 5 GHz Smart LED on/off Amazon Alexa One-button Fault Diagnosis WAN/LAN Port Auto-Negotiation |
| Other Function | IPTV IPv6 Guest Network System Log Remote Web Management MAC Clone Backup & Restore configurations Firmware upgrade online |
| Other Features |  |
| Temperature | Operating Temperature : 0℃~40℃ Storage Temperature : -40C ~70℃ |
| Humidity | Operating Humidity : 10 %~90% RH non-condensing Storage Humidity : 5 %~ 90%RH non-condensing |
| Certification | CE FCC RoHS |
| Package Content | BE3600 Mesh*3 Power Adapters *3 Quick Installation Guide *1 Ethernet Cable *1 NFC Sticker*1 |