5G06
AX3000 Wi-Fi 6 5G NR Router

| Hardware Specifications |  |
| --- | --- |
| Cellular Technologies | 5G/4G/3G Multi-Mode, 5G NR SA/NSA, 4G FDD/TDD LTE, 3G WCDMA |
| 5G NR 3GPP | Release 15 |
| 4G Category | DL：Cat18 , UL：Cat19 |
| 5G NR Peak Download Speed | SA: 4.67 Gbps, NSA: 3.74Gbps |
| 5G NR Peak Upload Speed | SA: 1.25 Gbps, NSA: 1.46Gbps |
| NR Frequency Band | 5G:n1,n3,n5,n7,n8,n20,n28,n38,n40,n41,n71,n77,n78 4G:B1,B3,B5,B7,B8,B20,B28,B32,B38,B40,B41,B42,B43 |
| CA combination | 5G NR:2CC; 5G EN_DC: 5CC |
| 5G NR Download MIMO | n1/n3/n7/n38/n41/n77/n78 4*4 MIMO |
| 5G NR Modulation | 256QAM |
| Wireless Connectivity | Wi-Fi 6 |
| Wi-Fi speed | 574Mbps on 2.4 GHz ; 2402Mbps on 5 GHz |
| Wi-Fi standards | 802.11b/g/n, 802.11ac, 802.11AX |
| Wi-Fi frequency | 2.4GHz, 5GHz |
| Wi-Fi transmit Power | 2.4GHz: up to 20dBm ; 5GHz: up to 23dBm |
| Ethernet Ports | 1 × 10/100/1000/2500 Mbps LAN/WAN Port 1 × 10/100/1000 Mbps LAN Port |
| SIM Card | 2 × Nano-SIM(4FF) Card Slots |
| Button | 1 × SIM Selection 1 × RST hole 1 × Mesh |
| Antennas and Ports | 8 × Internal 5G/4G Antennas 3 × Internal Wi-Fi Antennas 2 × TS-9 antennas ports |
| LED indicators | 1 × Internet indicator 1 × 5G/4G signal 1 × Wi-Fi indicator |
| Dimensions（L*W*H） | 92*118.5*196mm |
| Weight | Net weight: 500 grams Grass Weight: 860 grams |
| Power supply | Power adapter: Input power AC 100V-240V 50/60Hz Output power: DC 12V==2A |
| Software Specifications |  |
| WAN Type | 4G/5G or Wired |
| 5G/4G Features | Auto-APN 5G SA/NSA dual mode Auto dial-up 5G Location optimization 5G/4G E-UTRAN New Radio ‒ Dual Connectivity Roaming SMS(Send/Receive/Delete) USSD Data limted PIN Management |
| Wi-Fi functions | Broadcast and Hide Wi-Fi name Priority 5GHz Wi-Fi Guest network Wi-Fi schedule OFDMA MU-MIMO WPS |
| Wi-Fi security | WPA2/WPA3, WPA/WPA2, WPA2-PSK, WPA-PSK, WEP |
| Software features | Tenda XMesh Parental control DHCP(DHCP Server and Client) Virtual Server(UPnP/DMZ/Port Forwarding) DDNS(DynDns, 88ip) VPN(PPTP Client/L2TP Client/PPTP Server/IPSEC) Firewall QOS Upgrade ﬁrmware from local or OTA IPV4/IPV6 dual stack Local/remote management/APP TR069 |
| Working Environment |  |
| Temperature | Operating Temperature: 0℃~40℃ Storage Temperature: -30ºC~70ºC |
| Humidity | Operating Humidity: 10% ~ 90% RH Non-condensing Storage Humidity: 5% ~ 90% RH Non-condensing |
| Package lists |  |
| In the Box | 1 * 5G NR Router 1 * Ethernet cable 1 * Power adapter 1 * User Guide |