BE12 Pro
BE7200 Dual-Band Wi-Fi 7 Router with 2.5G Ports

| Caracteristici hardware |  |
| --- | --- |
| Standarde și Protocoale | IEEE802.3, IEEE802.3u, IEEE802.3ab |
| CPU | 4-Core 2GHz CPU |
| RAM | 512MB |
| Interfață | 2*10/100/1000/2500Mbps WAN/LAN ports 3*10/100/1000Mbps WAN/LAN ports |
| Antene | 5*external antennas |
| Butoane | 1*Mesh/WPS button, 1*Reset button |
| Alimentare | Input: AC 100-240V~50/60Hz, 0.5A Output: DC12V⎓2A |
| Indicatoare LED | 1* LED |
| Dimensiuni | (L*W*H) 290.25mm* 52.4mm*230mm |
| Caracteristici wireless |  |
| Standarde Wi-Fi | IEEE 802.11be/ax/ac/n/a 5 GHz IEEE 802.11be/ax/n/b/g 2.4 GHz |
| Benzi Wi-Fi | 5 GHz: 5765 Mbps (802.11be, HE160) 2.4 GHz: 1376 Mbps (802.11be) |
| Rată transfer Wi-Fi | 2.4G: 2412-2472MHz(CH1-CH13) 5G: 5150-5350MHz(CH36-CH64) 5725-5850MHz(CH149-CH165) |
| Alte caracteristici | CE: ＜20 dBm（2.4 GHz） ＜23 dBm（5 GHz） FCC: ＜30 dBm（2.4 GHz） ＜30 dBm（5 GHz） |
| Basic Features | SSID broadcasting and hiding MU-MIMO Beamforming signal directional transmission technology OFDMA Multi-Link Operation（MLO） 4K-QAM Multi-RUs |
| Protocoale securitate Wi-Fi | WPA2-PSK WPA3-SAE/WPA2-PSK Wireless Security: Enable/Disable WPS（WiFi Protected Set-up) fast encryption |
| Caracteristici software |  |
| Moduri de conectare la internet | PPPoE, Dynamic IP, Static IP PPPoE(Russia), PPTP(Russia), L2TP(Russia) Unify, Maxis, Maxis-special, Celcom, Digi |
| Protocol de rețea | IPv4&IPv6 |
| Moduri de lucru | Router Mode AP Mode Universal Repeater Mode（Client+AP,WISP） |
| DHCP | DHCP Server DHCP Client List DHCP Static IP Address Reservation and Allocation |
| Control parental | Client Filter URL Filter Time Limit |
| Opțiuni avansate redirecționare și acces | Port Forwarding DMZ Host UPnP |
| Firewall | ICMP Flood Attack Defense TCP Flood Attack Defense UDP Flood Attack Defense Block Ping From WAN |
| DDNS | DynDNS, no-ip, oray, 88ip |
| VPN | IPSec Transparent Transmission PPTP Transparent Transmission L2TP Transparent Transmission PPTP VPN Server PPTP VPN Client L2TP VPN Client OpenVPN Client WireGuard Client |
| Caracteristici speciale | WAN/LAN Port Auto-Negotiation EasyMesh Smart Diagnosis Wi-Fi timer switch Wireless power adjustment NFC IPTV IPv6 IoT Network Power Control Smart LED on/off |
| Alte caracteristici software | Support App and Web Management Bandwidth Control Guest Network MAC Address Clone Supports black list and white list Unify 2.4GHz & 5GHz Static Routing Remote Web Management Backup & Restore configurations Firmware upgrade online |
| Alte caracteristici |  |
| Certificări | CE, FCC,RoHS |
| Temperatură | Operating Temperature : 0℃~40℃ Storage Temperature : -40C ~70℃ |
| Umiditate | Operating Humidity: 10%~90% RH non-condensing Storage Humidity: 5%~90% RH non-condensing |
| Conținut pachet | Wi-Fi Router *1 Cat 6 Cable*1 NFC Sticker*1 Power Adapter*1 Quick Installation Guide*1 |