AX12 Pro
AX3000 Dual-Band Gigabit Wireless Router

| Hardware |  |
| --- | --- |
| Standard&Protocol | IEEE802.3, IEEE802.3u, IEEE802.3ab |
| Interfaces | 1× Gigabit WAN Port 3× Gigabit LAN Ports |
| Antenna | 4* High gain external antennas 5GHz:7dBi,2.4GHz:5dBi |
| Button | 1*WPS/Reset button |
| Power | Input：AC 100-240V~50/60Hz Output:DC 12V⎓1A |
| LED Indicator | 1* LED |
| Dimension | 226*144*168mm （L*W*H) |
| Wireless |  |
| Wireless Standard | IEEE 802.11ax/ac/n/a 5GHz IEEE 802.11ax/n/b/g 2.4GHz |
| Data Rate | 5 GHz: 2402 Mbps ( 802.11ax, HE160) 2.4 GHz: 574 Mbps ( 802.11ax) |
| Frequency | CE: 2.4G: 2412-2472MHz(CH1-CH13) 5G: 5150-5350MHz(CH36-CH64) FCC: 2.4G: 2412-2462MHz(CH1-CH11) 5G: 5150-5350MHz(CH36-CH64) 5725-5850MHz(CH149-CH165) |
| Transmission Power | CE: <20 dBm (2.4 GHz) <23 dBm (5 GHz) FCC: <30 dBm (2.4 GHz) <30 dBm (5 GHz) |
| Basic Features | SSID broadcasting and hiding MU-MIMO Beamforming signal directional transmission technology 160MHz OFDMA 1024-QAM |
| Wireless Security | WPA2-PSK WPA3-SAE/WPA2-PSK Wireless Security: Enable/Disable WPS(WiFi Protected Set-up)fast encryption |
| Software |  |
| Internet Connection Type | PPPoE, Dynamic IP, Static IP PPPoE(Russia), PPTP(Russia), L2TP(Russia) Unify,Maxis,Maxis-special,Celcom,Digi,Other Special ISPs (VLAN/IPTV) |
| Network Protocol | IPv4&IPv6 |
| Operating Mode | Router Mode AP Mode Universal Repeater Mode（Client+AP,WISP） |
| Parental Control | Client Filter URL Filter Time Limit AI Voice Control |
| DHCP | DHCP Server DHCP Client List DHCP Static IP Address Reservation and Allocation |
| Virtual Server | Port Forwarding, DMZ Host, UPnP |
| Firewall | ICMP Flood Attack Defense TCP Flood Attack Defense UDP Flood Attack Defense Block Ping From WAN |
| DDNS | DynDNS, no-ip,oray,88ip |
| VPN | IPSec Transparent Transmission PPTP Transparent Transmission L2TP Transparent Transmission PPTP VPN Server PPTP VPN Client L2TP VPN Client Open VPN Client WireGuard VPN Client |
| Special Features | Wi-Fi+(Mesh System) EasyMesh IoT Network Wireless Access Binding NFC Stickers AI Voice Parental Control Auto Upgrade Firmware Guest network Smart diagnosis Smart LED on/off Wi-Fi timer on/off WAN/LAN port auto-sensing |
| Other Function | Unify 2.4GHz &5 GHz IPTV IPv6 Bandwidth Control MAC Address Clone Smart Power Saving Black list and White list Support App and web management Remote Web Management Bandwidth Control MAC Address Clone Smart Power Saving Black list and White list |
| Others |  |
| Temperature | Operating Temperature : 0℃~40℃ Storage Temperature : -40C ~70℃ |
| Humidity | Operating Humidity : 10 %~90% RH non-condensing Storage Humidity : 5 %~ 90%RH non-condensing |
| Certification | CE、FCC、RoHS、UKCA |
| Package Contents | Wi-Fi Router *1 RJ45 Ethernet Cable*1 NFC Sticker*1 Power Adapter*1 Quick Installation Guide*1 |