3G150S-How to setup the router in Smart Mode？

Product line: 3G                 Question category: Internet settings

Q: What is a smart mode?

A: Depending on whether there is an available 3G modem, system automatically switches to

a) 3G Router mode (in the presence of a 3G dongle):clients can share Internet access either through a wireless or wired connection to the device. See connection diagram below.

b) Wireless Router mode (in the absence of a 3G dongle): device functions as a common wireless NAT router. Clients can only access device's web utility wirelessly to do the internet settings in wireless router mode. See connection diagram below.

The Steps are as below:

1. Two ways to connect to 3G150S

- Wired Network Connection

- Use an ethernet cable connect the 3G150S to your computer.

- Wireless Network Connection

- Search the Tenda wireless signal whose SSID entitled "Tenda_xxxxxx". If you don’t find it, please click "Refresh Network List". Right click on it and choose "Connect" and then enter the default security key “12345678” to connect.

- 2. Login to Web Utility

- Launch a web browser, in the address bar, input 192.168.2.1 and press "Enter".

Enter "admin" as a password and then click on Login.

You will see the page below.

- Check Smart Mode and click on Save. It will reboot automatically. After that 3G150S will recognize working mode depending on whether there is an available 3G modem.

- When the router works in 3G Mode, please refer to this document to do the settings.

- When the router works in Wireless Router Mode, please refer to this document to do the settings.

- (double click on it)