## 1  Two powerline Adapters establish the private network

- For example：Powerline Adapter 1, powerline Adapter 2, establishing the private network.

The steps of setting:

❶ Please keep pressing the Pair/Reset button of powerline adapter No.1 for 1~3 seconds, the PLC light will blink.

❷ Please keep pressing the Pair/Reset button of power adapter No.2 for 1~3 seconds in 2 minutes, the PLC light will blink.  When PLC light of two powerline adapters changes into normally on state, they establish the powerline network successfully.

## 2  The powerline Adapter adds the private network established.

- For example: If the powerline Adapter 1 and Adapter 2 have established the private network N1, you want to add the powerline Adapter 3 into N1.

The steps of setting:

❶ Please keep pressing the Pair/Reset button of the powerline Adapter 3 for 1~3 seconds, its  LED will blink, which means the powerline Adapter is checking the network N1.

❷	Please keep pressing the Pair/Reset button of the powerline Adapter 1 for 1~3 seconds in 2 minutes, its  LED will blink, which means they are pairing automatically.

The powerline Adapter 3 adds into the private network N1 with success when   LED should be on.

## 3  The powerline Adapter disconnects the private network

- For example：The powerline Adapter 1, Adapter 2 and Adapter 3 have established the private network, but the powerline Adapter 3 disconnects the private network.

The steps of setting:

Please keep pressing the Pair/Reset button of the powerline Adapter 3 for up 6 seconds, its  LED will blink for 3 times, which means it disconnects the private network and restores to the factory default setting.