How to setup the Dual-band router for DHCP Internet Connection Type?

Product line: Wireless   Question category: Internet settings

Step1 Connect the devices

Step2 Configure your computer（Generally you can skip this step, unless you can’t do step3）

Step3 Setup the router

Step4 Check whether you can access internet

Connect the modem to the blue port of the router, and then connect the router to your computer by a cable as the following picture.

Attention: After connection, the Power and Internet(WAN) light of the router should be blinking, 2.4G and 5G light should be on , one of LAN ports (12,3,4) is on.

Set your computer to obtain the IP address automatically

Click this link to see the whole process.

1.Open a browser or safari, then input 192.168.0.1 in the address bar, click Enter.

- Input admin, admin, click Login

If you can’t login to this page, please click this link.

- Choose DHCP, select 2.4G Security Key, then set an 8 character security key (wireless password) for it, Select 5G Security Key, then set the security key for it,too. The security key for 2.4G and 5G can be the same or different. Click OK to save the settings.

If you are not sure whether you should choose PPPOE or DHCP,please click this link.

If the connection status shows Connected, it means that you can have internet. Please check.

If the WAN Status shows Connecting, please do as the following steps.

- Just wait for one minute.

- Clone MAC Address ,then click Save as the following picture and then go back to Device Info to check WAN Status.

- Change the LAN IP address 192.168.0.1 to 192.168.5.1

4. After rebooting, please check the WAN Status, if it still shows Connecting, please power off the modem and the router for about 5 minutes and then power both then on.